Nissan Leaf ZE1

EV-CAN Monitor

ESP32-C3 + SN65HVD230 • ESPHome • Standalone Web Dashboard

# Overview

This project provides a standalone CAN bus monitor for the Nissan Leaf ZE1 electric vehicle. It reads live data directly from the EV-CAN network via the M101 gateway connector, decoding battery state of charge, voltage, current, temperature, charge status, and more. All data is served on a local web dashboard at a fixed IP address — no Home Assistant, no cloud, no dependencies.

| Component | Details |
| --- | --- |
| Microcontroller | ESP32-C3 Super Mini (Tenstar Robot) |
| CAN Transceiver | SN65HVD230 module (3.3V) |
| Firmware | ESPHome 2026.3.x |
| Power | 12V from M101 pin 3 via DC-DC to 5V |
| CAN tap | M101 pin 12 (CANH), pin 24 (CANL) |
| Web interface | http://192.168.1.99 (configurable) |
| Update method | OTA over WiFi after first flash |

# What It Reads

All data comes from EV-CAN broadcast frames — no queries required for the main sensors. The following signals are decoded using the dalathegreat EV-can_ZE1.dbc file.

| Sensor | Source Frame |
| --- | --- |
| LBC SOC (%) | 0x55B |
| Battery Voltage (V) | 0x1DB |
| Battery Current (A) | 0x1DB |
| Ambient Temperature (°C) | 0x54C |
| State of Health (%) | 0x5BC |
| GIDS remaining | 0x5BC |
| Discharge Power Limit (kW) | 0x1DC |
| Charge Power Limit (kW) | 0x1DC |
| Charge Status (Charging / Plugged In / Not Plugged In) | 0x1D4 + current |
| Gear Shift (Park / Reverse / Neutral / Drive) | 0x11A |
| Car Status (ON / OFF) | 0x11A |
| Connector detection | 0x1F2 |
| Time to 80% (min) | Calculated |
| Time to 100% (min) | Calculated |
| Estimated Battery Capacity (kWh) | Self-calibrating from charge sessions |

# Requirements

## Hardware

- ESP32-C3 Super Mini (Tenstar Robot variant)

- SN65HVD230 CAN transceiver module (3.3V)

- DC-DC buck converter (12V to 5V, at least 500mA)

- Wires and connector to tap M101 gateway

## Software (Windows)

- Python 3.12.x (not 3.13 or 3.14 — PlatformIO incompatibility)

- Git

- ESPHome

# Installation

## Step 1 — Install Python 3.12

- Go to python.org/downloads and download Python 3.12.x (latest 3.12 release)

- Run the installer

⚠️ Important: Tick 'Add Python to PATH' before clicking Install — easy to miss

- Verify in Command Prompt:

python --version

## Step 2 — Install Git

- Go to git-scm.com/download/win and download the installer

- Run it — all defaults are fine, just click Next through everything

- Close and reopen Command Prompt after install

## Step 3 — Install ESPHome

Open Command Prompt and run:

pip install esphome

Verify with:

esphome version

📝 Note: If 'esphome' is not recognised, use: py -3.12 -m esphome version

## Step 4 — Create project folder

Create a folder for your project, for example:

C:\esphome

Save the leaf-can.yaml configuration file into this folder with your WiFi credentials filled in.

# Configuration

Open leaf-can.yaml in a text editor (Notepad++ or VS Code recommended) and fill in your details:

| Setting | Where to change it |
| --- | --- |
| WiFi SSID | wifi: ssid: field |
| WiFi password | wifi: password: field |
| Static IP | manual_ip: static_ip: (default 192.168.1.99) |
| Gateway | manual_ip: gateway: (match your router IP) |
| OTA password | ota: password: field |

⚠️ Important: The ESP32 only supports 2.4GHz WiFi. If your router has separate 2.4GHz and 5GHz networks, use the 2.4GHz network name.

⚠️ Important: Make sure 192.168.1.99 is outside your router's DHCP range so no other device gets assigned that IP.

# First Flash (USB)

The first flash must be done via USB. After that all updates are wireless via OTA.

## Install USB driver

The ESP32-C3 Super Mini uses a CH340C USB chip. Install the driver from:

wch-ic.com/downloads/CH341SER_EXE.html

Plug in the ESP32-C3 and check Device Manager — you should see a COM port appear under Ports (COM & LPT).

## Flash via ESPHome dashboard

- Open Command Prompt and navigate to your project folder:

cd C:\esphome

- Start the ESPHome dashboard:

py -3.12 -m esphome dashboard .

- Open http://localhost:6052 in Chrome or Edge

- Your leaf-can.yaml will be listed — click Install

- Choose 'Plug into this computer'

- Select the COM port for your ESP32-C3

- Wait for compilation (2-3 minutes first time) and flashing

## Bootloader mode (if flashing fails)

If the ESP32-C3 doesn't accept the flash, put it into bootloader mode manually:

- Hold the BOOT button on the board

- Press and release RESET

- Release BOOT

- Try flashing again immediately

# OTA Updates (Wireless)

After the first USB flash, all future updates are done wirelessly over WiFi. The ESP must be powered and connected to your network.

## Update via dashboard

- Start the dashboard if not already running:

cd C:\esphome

py -3.12 -m esphome dashboard .

- Open http://localhost:6052

- Edit your YAML if needed

- Click Install → Wirelessly

- The ESP will download and apply the update automatically

## Update via command line

Alternatively, compile and push in one command:

py -3.12 -m esphome run leaf-can.yaml

When prompted, choose the wireless/OTA option.

📝 Note: OTA requires the ESP to be reachable at its static IP. If it's not responding, check your router's device list to confirm it's connected.

# Hardware Wiring

## SN65HVD230 to ESP32-C3

| SN65HVD230 pin | ESP32-C3 pin |
| --- | --- |
| TXD | GPIO4 |
| RXD | GPIO5 |
| VCC | 3.3V |
| GND | GND |
| R (termination) | Leave floating — do not connect |

⚠️ Important: The SN65HVD230 is a 3.3V device. Never connect VCC to 5V or 12V.

⚠️ Important: Do not add a termination resistor between CANH and CANL. The Leaf's bus already has terminators at both ends.

## M101 Gateway Connector (car side)

| M101 pin | Connection |
| --- | --- |
| Pin 12 (CANH, no wire colour listed) | SN65HVD230 CANH |
| Pin 24 (CANL, no wire colour listed) | SN65HVD230 CANL |
| Pin 3 (White, 12V Battery) | DC-DC converter input + |
| Pin 5 (Black, GND) | DC-DC converter input − |

The DC-DC converter output (5V) connects to the ESP32-C3 5V/VIN pin. The ESP32-C3 then provides 3.3V to the SN65HVD230.

📝 Note: Pin numbers are from the harness side, looking into the back of the connector from the wire side.

# Viewing Data

Once the ESP is wired to the car and powered, open a browser on any device connected to your WiFi network and go to:

http://192.168.1.99

The page shows all sensor values and updates automatically. No login required.

## Charge monitoring

The monitor automatically detects charging by combining the plug status from 0x1D4 with actual current flow from 0x1DB. Charge Status shows one of four states:

- Charging — plug connected and current > 0.5A

- Plugged In — plug connected but no current flowing (charge complete or paused)

- Interrupted — charge was interrupted

- Not Plugged In — no plug detected

## Time to charge estimates

Time to 80% and Time to 100% are calculated from current charge power (voltage × current) and estimated battery capacity. The capacity estimate self-calibrates over multiple charge sessions — after the SOC rises 5% in a single session the actual capacity is calculated and saved to flash. The estimate improves with each charge.

## Wake Car button

When the car is off and unplugged the CAN bus goes silent and no data is available. The Wake Car button sends a wake-up frame (0x56E) which may prompt the LBC and other modules to resume broadcasting. Results vary depending on car state.

# Logging for Debugging

To capture logs to a file for debugging, run the following in Command Prompt:

chcp 65001

py -3.12 -m esphome logs leaf-can.yaml > leaf-can-log.txt 2>&1

Press Ctrl+C when done. Open leaf-can-log.txt in Notepad to view all log output.

📝 Note: chcp 65001 switches the console to UTF-8 encoding which is required to avoid errors from the degree symbol in the temperature sensor name.

# Power Consumption

| Mode | Approximate current |
| --- | --- |
| Normal operation (WiFi connected, CAN active) | ~50mA at 3.3V |
| WiFi association (brief, on boot) | ~200mA peak |
| Standby draw from 12V via DC-DC | ~20mA at 12V |

At 20mA from the 12V battery, over 3 days the device draws approximately 1.44Ah — well within the Leaf's 12V auxiliary battery capacity. The car's own standby systems draw significantly more than this.

# Credits and References

- ESPHome firmware framework: esphome.io

- EV-CAN DBC file: dalathegreat on GitHub (EV-can_ZE1.dbc)

- Original ESPHome Leaf integration concept: blog.jingo.uk

- M101 gateway connector pinout: Nissan Leaf community documentation

- OVMS (Open Vehicle Monitoring System): docs.openvehicles.com